IEEE视频技术电路与系统汇刊，第30卷，第6期，2020年6月

基于神经网络的图像和视频压缩研究进展

马思伟，IEEE成员，Zhang Xinfeng Zhang，IEEE成员，Chuanmin Jia，IEEE学生成员，

赵振辉王世琦，IEEE成员，王珊舍

摘要

索引术语

原稿于2018年10月10日收到；于2019年2月2日和2019年3月10日修订；于2019年4月1日接受。出版日期：2019年4月17日；现行版本日期：2020年6月4日。这项工作部分地由中国国家自然科学基金在资助61632001和赠款61571017的支持下，部分由香港RGC早期的事业计划9048122资助，部分由CITYU授予21211018，部分由Peng Cheng Lab.推荐副编辑H. Schwarz。（通讯作者：马思伟） S、 Ma，C.Jia，Z.Zhao和S.Wang在北京大学电子工程与计算机科学学院数字媒体研究所工作，中国北京100871，中国，深圳市鹏程实验室，邮编518055（电子邮件：swma@pku.edu.cn；cmjia@pku.edu.cn；zhzhao@pku.edu.cn；sswang@pku.edu.cn). 十、 张是中国科学院大学计算机科学系，北京100049（电子邮箱：zhangxinf07@gmail.com网站). S. Wang在香港香港大学城计算机系工作（电子邮件：shiqwang@cityu.edu.hk). 本文中一个或多个图形的彩色版本可在http://ieeexplore.ieee.org。 数字对象标识符10.1109/TCSVT.2019.2910119

一、 简介

我

在传输网络和存储能力有限的情况下，MAGE和视频压缩在提供高质量图像/视频服务方面发挥着重要作用。图像和视频中的冗余对于图像和视频压缩至关重要，包括空间冗余、视觉冗余和统计冗余。此外，视频序列中存在的时间冗余使得视频压缩比图像压缩具有更高的压缩比。

对于图像压缩，早期的压缩方法主要是直接利用熵编码来减少图像中的统计冗余，如Huffman编码[1]、Golomb编码[2]和算术编码[3]。在20世纪60年代后期，通过对空间频率进行编码来实现图像压缩的变换编码，包括傅里叶变换[4]和哈达玛变换[5]。1974年，艾哈迈德等人。提出了一种用于图像编码的离散余弦变换（DCT）[6]，它可以在低频域压缩图像能量，从而使频域压缩变得更加高效。

除了通过熵编码和变换技术来减少统计冗余外，还进一步提出了预测和量化技术来减少图像中的空间冗余和视觉冗余。目前最流行的图像压缩标准JPEG是一种成功的图像压缩系统，它综合了已有的编码技术。它首先将图像分成块，然后将块变换到DCT域。对于每个块，将差分脉冲编码调制（DPCM）[7]应用于其DC分量，从而压缩相邻DCT块之间DC分量的预测残差，而不是直接压缩DC值。为了减少视觉冗余，我们设计了一个特殊的量化表，可以很好地保存低频信息，并且由于人类对高频部分的信息丢失不太敏感，因此丢弃了更多的高频（类噪声）细节[8]。另一个著名的静止图像压缩标准jpeg2000[9]采用2D小波变换代替DCT以紧凑的形式表示图像，并利用高效的算术编码方法EBCOT[10]来减少小波系数中存在的统计冗余。

1051-8215©2019 IEEE。允许个人使用，但重新发布/重新分发需要IEEE许可。看到了吗https://www.ieee.org/publications/rights/index.html了解更多信息。

在视频编码中，可以通过帧间预测消除的时间冗余成为视频编码的主流

图1。HEVC帧内预测说明。（a） 不同的帧内模式；（b）角度预测实例。

由于在很短的时间间隔内捕获的连续帧之间的高度相关性。为了有效地获得帧间预测，20世纪70年代提出了基于块的运动预测[11]。1979年，Netravali和Stuller提出了运动补偿变换框架[12]，被称为当今的混合预测/变换编码器。读者介绍了第一代方法的历史发展[13]。

经过几十年的发展，预测/变换混合编码方法已经取得了很大的成功。许多编码标准已经开发并广泛应用于各种应用，如MPEG-1/2/4、H.261/2/3和H.264/AVC[14]，以及AVS（中国音频和视频编码标准）[15]和HEVC[16]。以最新的视频编码标准HEVC为例，利用相邻重建像素预测当前编码块，有33种角度帧内预测模式、DC模式和平面模式，如图1所示。对于帧间编码，HEVC通过从多个角度进一步细化其前身H.264/AVC来提高编码性能，例如增加PU除法的多样性，利用更多的内插滤波器抽头进行子样本运动补偿[17]，并改进侧信息编码，包括用于帧内模式编码的更可能模式（MPMs）[18]、用于运动矢量预测器编码的高级运动矢量预测（AMVP）和合并模式[19]。在最先进的视频编码框架中，另一种新的视频编码工具是循环滤波，自2000年以来，人们提出了许多环路滤波器[20]-[25]。在本文中，采用了[He-offset]中的[He-offset]和[He-offset]进行了[He-offset]的自适应滤波。然而，传统的基于图像和视频局部相关性的混合视频编码框架的细化策略越来越难以进一步提高编码效率。

近年来，神经网络，特别是卷积神经网络（CNN）在图像/视频理解、处理和压缩等领域取得了巨大的成功。特别是，有些任务还会在卷积层之后附加几个完全连接的层。这些层中的参数可以通过端到端的策略，基于为特定任务标记的大量图像和视频样本进行良好的训练。利用CNN对数据进行有效的分类测试，可以很好地解决分类问题。CNN产生的预测信号的质量已经超过了基于规则的预测。此外，CNN可以解释为特征抽取器，将图像和视频转换成具有紧凑表示的特征空间，有利于图像和视频的压缩。基于CNN的这些优良特性，它也被认为是一种很有前途的压缩任务解决方案。因此，为了更好地了解CNN在图像和视频压缩方面的现有发展，本文对基于神经网络的图像和视频压缩进行了详细的综述。

鉴于本文的研究范围之广，我们将论文的主体分为四个部分，以便更清楚地呈现。在第二节中，我们介绍了神经网络和图像/视频压缩的基本概念。第三节详细回顾了基于神经网络的图像压缩技术的发展。在第四节中，我们回顾了基于神经网络的视频压缩技术。在第五节中，我们回顾了基于神经网络的图像和视频压缩优化技术。第三节的进一步理论基础主要遵循网络发展的时间表，介绍基于典型网络结构的基于神经网络的图像压缩。在第四节中，我们主要介绍了嵌入到最新的混合视频编码框架HEVC中的基于CNN的视频编码技术，并介绍了一些基于CNN的新的视频编码框架。最后，第六部分展望了基于深度学习的图像/视频压缩面临的重要挑战，并对论文进行了总结。

二。神经网络与图像/视频压缩简介

在这一部分中，我们首先简要回顾了神经网络的基本概念和发展历史。接着介绍了基于块的图像编码和混合视频编码框架的框架和基本技术的发展。

A、 神经网络

随着神经科学和数学的交叉研究，神经网络（NN）应运而生，它在非线性变换和分类方面表现出强大的能力。直观地说，网络由多层简单的处理单元组成，这些单元通过加权连接相互作用。神经元通过先前激活的神经元的加权连接被激活。为了实现非线性，激活函数总是应用于所有中间层[29]。一个简单的神经网络结构如图2所示，它由一个输入层、一个输出层和多个隐藏层组成，每一层包含不同数量的神经元。

简单感知器的学习过程是20世纪60年代提出并分析的[30]。20世纪七八十年代，在训练目标导数的链式法则的启发下，提出了反向传播法[31]、[32]，以解决多层感知器（MLP）的训练问题。然后，多层体系结构

图2。神经网络结构的说明。

虽然计算量大且局部极小值较差，但大多采用随机梯度下降法训练。然而，神经网络中相邻层之间的紧密连接使得模型参数的数量呈二次方增长，阻碍了神经网络在计算效率上的发展。随着MLP（1990）中参数共享的引入，提出了一种更轻量化的卷积神经网络，并将其应用于文档识别中，使大规模的神经网络训练成为可能。

B、 图像和视频压缩

在各种编码框架中，图像和视频压缩的核心技术是变换和预测。JPEG[34]是最流行的图像压缩标准，它由基本的变换/预测模块组成，如图3（a）所示。在JPEG中，输入图像被分成8×8个非重叠块，每个块用块DCT（BDCT）变换到频域。对于每个变换后的块，DCT系数通过量化和熵编码压缩成二进制流。对于视频压缩，大多数流行的视频编码标准都采用如图3（b）所示的基于变换预测的混合视频编码框架，例如MPEG-2、H.264/AVC和HEVC。与JPEG不同，HEVC在空间域中利用了更多来自相邻重构块的帧内预测模式，而不是DC预测，如图1所示。除了帧内预测外，视频压缩的更多编码增益来自高效的帧间预测，即利用运动估计找到最相似的块作为待编码块的预测。另外，HEVC采用了去块滤波和SAO两种环路滤波器，依次减小压缩伪影。

在上述基于块的图像和视频编码标准中，压缩通常是块相关的，必须按块顺序执行，这限制了GPU等并行计算平台的压缩并行性。此外，与端到端优化压缩相比，每个编码工具的独立优化策略也限制了压缩性能的提高。本质上，还有另一种基于用于图像和视频压缩的神经网络技术的技术发展轨迹，如图4所示。随着神经网络的兴起，传统的图像/视频压缩与CNN的结合进一步推动了它们的发展。在接下来的章节中，我们将介绍基于神经网络的图像/视频压缩的发展和相关的代表性技术。

三、 基于神经网络的图像压缩研究进展

在这一部分中，我们介绍了主要起源于20世纪80年代末[35]的基于神经网络的图像压缩方法。本节根据神经网络技术的历史发展进行组织，主要包括多层感知器（MLP）、随机神经网络（Random neural network，MLP）、卷积神经网络（CNN）和递归神经网络（RNN）。在最后一节中，我们将介绍使用产生式对抗网络（generative atteriral networks，GAN）的图像编码技术的最新发展。

A、 基于多层感知器的图像编码

MLP[36]由神经元的输入层（或节点、单元）、几个隐藏的神经元层和最后一层输出神经元组成。每个神经元的输出hi

MLP表示为，

你好= σ(Nj=1威斯康星州ij公司+ ci),（一）

其中·）为激活函数，ci表示线性变换的二阶项，而表示可调参数，权重，表示层间的联系。理论分析表明，用一个以上的隐层构造的MLP可以将任何连续的可计算函数逼近到任意精度[37]。此属性为维度缩减和数据压缩等场景提供证据。利用MLP进行图像压缩的主要目的是为整个空间数据设计酉变换。σ(wij公司

1988年，Chua和Lin提出了一个端到端的图像压缩框架，它利用了神经网络的高并行性和强大的紧凑表示[35]，这可能是人脑类编码功能的模型。他们将传统的图像压缩步骤，即空域图像数据的酉变换、变换系数的量化和量化系数的二进制编码，作为一个综合优化问题，以最小化以下代价函数，

ek,我uu公司千吨我2      （2） 是的ˆ吉隆坡= (s12−1 + s22−2 + ··· +s吉隆坡2−b吉隆坡)−米k,我, （三）

其中yˆ∈[01]是重构的变换系数，uk是正交变换核，{··，}是表示yˆ量化水平的二进制码。然后，利用一个分解/决策神经网络来求解Eqn中的优化问题。（2） 寻找最佳的二进制码组合，即压缩比特流的输出。1989年，16个隐单元的全连通神经网络被训练成使用反向传播压缩图像的每个8×8块[39]。但是，这种策略固定了特定的神经网络参数吉隆坡,s1,s2,sbkl公司吉隆坡

图3。图像和视频压缩框架（a）JPEG压缩，（b）混合视频压缩。 图4。基于神经网络的压缩算法的技术路线图。

图5。基于神经网络的图像编解码器[38]。

二进制码的个数，很难适应可变压缩比的最优状态。

Sonehara等人。提出训练降维神经网络对输入图像进行压缩，并将量化和熵编码作为单独的模块[38]。图5显示了降维神经网络的结构，其中部署了自动编码器瓶颈结构。特别是瓶颈层的神经元数小于输入输出层的神经元数，从而降低了数据的维数。为了加快学习速度，将输入图像分成块，并将这些块并行地输入到不同的子神经网络中。多个子神经网络的设计要求输入图像与学习图像严格相似，因为不同的子神经网络负责纹理特定的结构。因此，该模型的推广是有限的，特别是未学习图像的信噪比损失高达10分贝。为了获得更好的性能和泛化能力，Sicuranza等人。训练了一个独特的小神经网络，按顺序给图像块输入，从未学习的图像到学习的图像的信噪比损失只有1dB左右[40]。

然而，上述算法的自适应性是通过手动设置不同数量的隐层神经元而不是引入具有更多层次和复杂连接的网络来决定的，这可能会限制MLP在压缩性能方面的能力[41]。为了解决这一问题，研究了基于MLP的预测图像编码算法[42]。具体地，采用向左和上方的空间信息（点A、B和C，每个小块对应于图6（A）中的一个像素）来生成图6（A）中右下角像素X的非线性预测器。MLP预测器有三个输入节点、30个隐藏节点和一个输出节点，如图6（b）所示，并且利用反向传播算法[32]训练MLP模型，以最小化原始信号和预测信号之间的均方误差。实验结果表明，与线性预测器相比，基于MLP的非线性预测器将误差熵从4.7bpp提高到3.9bpp。

压缩[42]。

为了进一步提高预测精度，Manikopoulos采用了方程（4）中的高阶预测模型，该模型能够很好地处理图像中边界和轮廓等定义清晰的结构[43]。

xn公司= − + − −我wx公司我（n）我)ij公司wx公司ij公司（n）我)十（n）j)

+伊吉克wx公司伊吉克（n）−我)十（n）−j)十（n）−k)+ ... + n,

（四）

其中n是零均值i.i.d.随机变量的序列。1996年提出了基于多层神经网络的分层训练算法（NTA），大大缩短了训练时间。感兴趣的读者可以参考[45]、[46]了解更多基于MLP的图像压缩技术，这些技术通过设计不同的连接结构来提高压缩效率。

B、 基于随机神经网络的图像编码

1989年引入了一类新的随机神经网络[47]。随机神经网络的性能不同于上述基于MLP的方法，其中信号在空间域中，并通过梯度反向传播方法进行优化。随机神经网络中的信号以单位幅度的尖峰形式传输。这些神经元之间的通讯被模拟成一个泊松过程，其中正信号代表兴奋信号，负信号代表抑制信号。文[47]给出了随机神经网络行为分析的一些理论结果。采用“反向传播”式训练方法进行参数更新，每次都需要用一个新的输入输出对求解n个线性方程组和n个非线性方程组。

一些研究者考虑了随机神经网络与图像压缩的结合，并给出了一些有意义的结果。Gelenbe和Sungur首先将随机神经网络应用于图像压缩任务[48]。该结构采用一个中间层的前馈编解码随机神经网络。具体来说，第一层以图像作为输入，最后一层输出重建图像，中间层生成压缩比特。Gelenbe和Sungur通过设计自适应静态逐块随机神经网络压缩/解压[49]，进一步扩展了[48]中的工作。有多个不同的神经压缩网络C1，…，CL设计来实现不同的压缩水平。每个

图7。[52]中提出的基于CNN的端到端图像压缩的参数化结构。

这些网络并行压缩块，并根据解压缩结果的质量来选择网络。Hai通过将随机神经网络集成到图像的小波域中，进一步提高了压缩性能[50]。

C、 基于卷积神经网络的编码

最近，CNN在图像分类、目标检测等高级计算机视觉任务中的表现远远超过了传统算法[51]。即使对于许多低层次的计算机视觉任务，它也能获得令人印象深刻的性能，例如，超分辨率和压缩伪影的减少。CNN采用卷积运算来表征相邻像素之间的相关性，级联卷积运算很好地符合自然图像的层次统计特性。此外，卷积运算引入的局部感受野和共享权值也降低了CNN的可训练参数，显著降低了过拟合问题的风险。受CNN对图像的强大表示的启发，人们进行了大量的工作来探索基于CNN的有损图像压缩的可行性。

然而，将CNN模型直接应用到端到端图像压缩中是很困难的。一般来说，CNN训练依赖于bp算法和随机梯度下降算法，该算法要求损失函数对卷积权重和偏差等可训练参数几乎处处可微。由于图像压缩中的量化模块，它几乎处处产生零梯度，从而阻止了CNN中参数的更新。另外，经典的率失真优化方法很难应用到基于CNN的压缩框架中。这是因为CNN的端到端训练需要一个可微的损失函数，但是速率的计算必须基于整个量子化单元的总体分布，这在CNN中通常是不可微的。

Ballé等人。2016年首次在标量量化假设下引入了端到端优化的CNN图像压缩框架[52]，[53]。该框架如图7所示，由两个模块组成，即编码器和解码器的分析和综合变换。在分析变换中，有卷积、子抽样和除法归一化三个阶段。每个阶段从仿射卷积开始：

五（米）,n)（香港）,ij公司米,n)+ ck公司,我,                    （五）

其中u是第k级在空间位置的第j个输入信道（米）,n)，*表示二维卷积运算，hk表示卷积参数。cks是卷积神经网络的偏差参数。卷积的输出被降采样：,ij公司,我

w我(k)（米）,n)=v我(k)（第公里,s千牛),（六）

其中sk是下采样因子。最后，由广义除法归一化（GDN）变换处理的下采样信号：

美国我(k+1)（米）,n)=w我（k）)（米）,n)1 .        （七）

(βk,我+ j（米）,n))2)2

其中和是标准化操作的偏差和比例参数。βk,我γk,ij公司

由于合成变换是分析变换的逆运算，因此将根据率失真目标函数，以端到端的方式对所有三个阶段的参数{}进行优化。为了处理量子化引起的零导数，Balléet al。在CNN训练过程中，利用加性i.i.d均匀噪声来模拟量化器，实现了随机梯度下降法求解优化问题。从PSNR和MS-SSIM指标来看，该方法优于JPEG2000。此外，Ballé和他的同事利用熵估计的标度超先验[54]对该模型进行了扩展，实现了与HEVC相似的目标编码性能。Minnen等人。继续增强熵编码的上下文模型，用于端到端优化图像压缩[55]，并优于HEVC帧内编码。由于自回归元件不易并行化，因此对于未来的实际应用，无论是硬件端支持还是能量效率分析都需要进一步的探索。周等人进一步改进了图像压缩性能。通过在编码器处使用金字塔特征融合结构，在解码器处使用基于CNN的后处理滤波器[56]。另一种结合量化和熵编码的端到端图像压缩工作可以参考[57]和[58]，基于CNN预测的图像压缩可以参考[59]。h,c,β,γ

D、 基于递归神经网络的编码

与上述CNN结构不同，RNN是一类具有记忆功能的神经网络，用于存储最近的行为。具体地说，RNN中的存储器单元具有与其自身的连接，它们传输过去执行时转换的信息。通过利用这些存储的信息，RNN改变当前转发进程的行为以适应当前输入的上下文。Hochreiter等人。为了克服衰减误差倒流的不足，提出了长短时记忆（LSTM）[60]。更先进的改进，如门控递归单元（GRU）[61]，以简化

图8。我们共享RNN架构的一次迭代[63]。

递归进化过程，同时保持了递归网络在相关任务中的性能[62]。与CNN类似，对于图像压缩任务，RNN仍然存在着速率估计梯度传播的困难。

Toderici等人。首先提出了一种基于RNN的图像压缩方案[63]，该方案利用比例加性编码框架来限制编码比特数，而不是CNN[52]中速率估计的近似值。更具体地说，文献[63]中提出的方法是一种支持渐进式变比特率压缩的多迭代压缩体系结构。如图8所示，在一次迭代中有三个模块，即编码网络e、二进制器B和解码网络D，其中D和e包含循环网络组件。输入图像块与解码网络D重建图像块之间的残余信号可以进一步压缩到下一个迭代中。为了进一步改进基于RNN的图像压缩，Minnen等人。提出了一种空间自适应图像压缩框架[64]。在该框架中，输入图像被分成块，这与现有的JPEG和JPEG2000等图像编解码器类似。对于每个块，由完全卷积神经网络从在左上方区域解码的空间相邻块生成初始预测。然而，根据已发布的结果，该方法的性能仅优于JPEG，而低于JPEG2000。

E、 基于生成对抗网络的编码

生成对抗网络是深层神经网络应用中最具吸引力的改进之一。GAN同时优化了发生器和鉴别器两种网络模型。鉴别器利用深层神经网络来区分样本是否从发生器中产生。同时，训练发生器克服甄别器，产生通过检测的样本。对抗性丢失有助于生成者提高图像的主观质量，也可以针对不同的任务进行设计。在图像压缩任务中，一些研究工作侧重于解码图像的感知质量，并利用GAN来提高编码性能。

代表性作品之一是Rippel和Bourdev于2017年提出的[65]，它是一个综合性和

图9。基于GAN的图像压缩的总体结构[65]。

经过优化的GAN基图像压缩，不仅压缩比得到了惊人的提高，而且利用GPU的大规模并行计算内核，可以实时运行。如图9所示，输入图像通过网络作为压缩形式压缩到非常紧凑的特征空间中，并且利用生成网络从特征重构解码图像。基于GAN的图像压缩与基于CNN或RNN的压缩方案的最大区别在于引入了对抗性损失，显著提高了重建图像的主观质量。生成网络和对抗网络通过联合训练来显著提高生成模型的性能。[65]中基于GAN的方法实现了显著的压缩比改进，例如，在所有质量级别上生成比JPEG和JPEG2000小2.5倍、比WebP小2倍、比BPG小1.7倍的压缩文件。在此，使用MS-SSIM来测量质量，而使用PSNR度量方法仍然不有效。受基于GAN的视图合成技术的启发，光场（LF）图像压缩可以通过使用LF中的采样上下文视图生成缺失视图来获得显著的编码增益[66]。特别是，GAN生成的内容比特定的纹理更符合原始内容的语义。特别是在放大重建图像时，可以看到特定纹理的内容差异。

此外，Gregor等人。在图像压缩任务中引入了一个齐次深生成卷积模型DRAW[67]。不同于以往的著作，格雷戈等。旨在通过生成尽可能多的图像语义信息进行概念压缩[68]。详细探讨了一种基于GAN的极端图像压缩框架，其目标比特率低于0.1bpp，允许不同程度的内容生成[69]。目前，基于GAN的压缩方法在人脸等窄域图像压缩中取得了成功，但对于一般自然图像模型的建立还需要进一步的研究。

四、 神经网络视频编码研究进展

利用最新的视频编码标准HEVC对基于深度学习的视频编码进行研究，是近年来研究的热点。HEVC中几乎所有的模块都通过整合各种深度学习模型进行了探索和改进。在这一部分中，我们将从HEVC的五个主要模块，即帧内预测、帧间预测、量化、熵编码和循环滤波，回顾了基于深度学习模型的视频编码的发展。最后，我们将介绍几种不同于混合视频编码框架的新的视频编码模式。

A、 基于神经网络的帧内预测技术

虽然已经提出了许多基于神经网络的图像压缩方法，可以作为视频压缩的内编码策略，但其性能仅优于JPEG和JPEG2000，明显低于HEVC帧内编码。这也显示了混合视频编码框架的优越性。因此，将神经网络技术引入到混合视频编码框架中，特别是将其与最先进的HEVC框架相结合，提高视频编码性能成为研究的热点。崔等。提出了一种内预测卷积神经网络（IPCNN）来提高帧内预测效率，这是首次将CNN与HEVC帧内预测相结合的工作。在IPCNN中，首先根据HEVC帧内预测机制对当前的8×8块进行预测，由模式决策生成的当前块的最佳预测版本及其作为附加上下文的三个最近邻重构的8×8块组成一个16×16块，作为IPCNN的输入。采用残差学习方法，通过从输入块中减去原始块，IPCNN的输出就是残差块。然后，可以通过对输入块8减去一个块中的剩余块8，得到该块的内部预测。所设计的IPCNN不仅继承了CNN强大的预测效率，而且利用了空间相邻8×8块的远距离结构信息，而不是仅仅利用一列一行重建的相邻像素作为HEVC帧内预测。预测的附加上下文以及剩余学习方法提供了额外的编码效率。

不是使用CNN来提高最佳HEVC帧内预测的质量，Li等人。提出了一种新的全连通网络帧内预测模式[70]，它与现有的35种HEVC帧内预测模式进行了竞争。与IPCNN类似，IPFCN也利用重建像素的相邻多条参考线作为上下文输入，但是没有利用HEVC帧内预测的当前块的预测版本。图10示出了IPFCN结构，其是从重构的相邻像素到当前块的端到端帧内预测映射。除了输出层外，每个连接层后面都有一个非线性激活层，其中利用了参数校正线性单元（PReLU）。输出层的每个节点对应一个像素。相应的编码性能和复杂度如表所示。一、 其中，缩写“L”表示轻（表示不带“L”的模型的参数简化版本），“D”表示双重（即直流模式和平面模式的一列特定IPFCN模型，

图10。IPFCN的网络结构[70]。

表一

IPFCN[70]在全长序列通用测试条件下的编码性能

另一个IPFCN模型用于剩余的角模式），而“S”表示单个模型（即为所有帧内预测模式训练一个模型）。在CPU平台上进行了运行时间测试。与HEVC参考软件HM-16.9相比，该方法实现了明显的比特率节约，平均节省了3.0%的BD速率。尤其是，IPFCN对于A级超高分辨率4K视频的性能更好，实现了高达4.4%的BD速率节省。但是，由于全连通的神经网络和乘法运算中的浮点运算，复杂度极高，如表一所示，解码速度提高了200倍以上，文[71]提出了基于CNN的色度帧内预测，利用重建的luma块和相邻的色度块来提高帧内色度预测效率。在[72]中，Pfaff等人。在JEM软件下提出了一种更高效的帧内预测网络，简化后的帧内编解码过程运行时间仅增加74%和38%，BD速率节省约2.26%。

Li等人没有使用相邻的参考样本来获得块预测。探讨了基于CNN的下/上采样技术作为HEVC的一种新的帧内预测模式，并在[74]中提出了对帧间的扩展。不同于以往的图像水平下/上采样技术[75]，[76]，Li等人。设计了CTU级的下/上采样方法，框架如图11所示。在下/上采样模式下，首先对每个CTU进行低分辨率下采样，然后采用HEVC帧内编码方法进行编码。对重建后的低分辨率CTU进行上采样，以恢复其原始分辨率。要移除边界伪影，请稍等

图11。基于上采样的神经网络帧内预测框架[73]。

对整个帧进行重构时，采用阶段上采样的CNN网络。然后，第二级上采样CNN可以访问下采样/上采样ctu周围的所有块。为了保证编码效率，在码流中发送一个标志来指示下采样/上采样是否开启。在此，根据编码器处的速率失真优化来确定标志。由于基于CNN的上采样技术的高效性，本工作在低比特率情况下获得了显著的编码增益，与HEVC相比平均节省了约5.5%的比特率。然而，由于上采样算法的局限性，对于luma分量，HEVC常用测试条件下使用的QPs（=22、27、32、37）的比特率节省仅为0.7%。

此外，压缩训练视频序列中的QPs也会影响性能，当测试QPs偏离训练阶段的QPs时，性能会退化。基于使用交叉QP模型的结果，对于从QP=”处压缩的视频中学习到的模型，将其应用于在QP0+2处压缩的视频时的性能退化小于应用于在QP0−2处压缩的视频的性能退化。结果表明，下/上采样CNN预测模型对高QPs压缩的视频具有较强的鲁棒性。除了帧内预测，Pfaff等人。利用具有一个隐层和相邻重构样本的全连通神经网络来预测内模概率[77]，这对熵编码模块是有益的。QP公司0

为了减轻压缩噪声对上采样CNN的影响，我们提出了一种基于双网络的超分辨率策略，通过使用增强网络连接低分辨率图像和上采样网络[78]。增强网络专注于压缩噪声的降低，并向上采样网络提供高质量的输入。与单上采样网络相比，该方法进一步提高了低比特率情况下的编码性能，特别是对于超高分辨率视频。2019年，Li等人。设计了一个紧凑的表示CNN模型，通过限制低分辨率图像的信息丢失，进一步改进了基于CNN的超分辨率压缩框架[79]。其他基于CNN的帧内编码技术可以参考[80]，[81]，其中[80]中引入了用于帧内编码的CTU级CNN增强模型，在[81]中引入了基于RNN的利用相邻重构样本的帧内预测。

图12。[84]中FRCNN的框架。

B、 基于神经网络的互预测

In hybrid video coding, the inter prediction is realized by motion estimation on previous coded frames against the current frame, and in HEVC the precision of motion estimation is up to quarter-pixel, the value of which is calculated via interpolation, e.g., discrete cosine transform based interpolation filter (DCTIF) [17]. Intuitively, the more similar of the inter predicted block and the current block are, the higher coding performance is achieved due to fewer prediction residuals left. Huo et al. proposed a straightforward method [82] to improve inter prediction efficiency by utilizing the existing variable-filter-size residue-learning CNN (VRCNN) [83], which is named CNN-based motion compensation refinement (CNNMCR). The CNNMCR jointly employs the motion compensated prediction and its neighboring reconstructed blocks as input of VRCNN, which is trained by minimizing the mean square errors between the input and its corresponding original signal. In fact, the improvement of CNNMCR for inter prediction is because the designed network can improve the inter prediction quality by reducing the compression noise and the boundary artifacts due to independent block processing.

Considering the importance of fractional-pixel motion compensation in inter prediction, Yan et al. proposed a Fractionalpixel Reference generation CNN (FRCNN) to predict the fractional pixels [84]. This work is different from the previous interpolation or super-resolution problems, which predict pixel values in high resolution image, while FRCNN is to generate the fractional-pixels from reference frame to approach the current coding frame. Therefore, the fractional-pixel generation is formulated as a regression problem with the loss function as,

f ∗ ≡ argminL(8) ∈( f (X),Y), f

where X is the motion compensation block by integer motion vector, Y is current coding block, and f is the regressor, which is implemented by CNN. Since the optimal position indicated by motion vectors may be in different fractional-pixel positions, e.g., three positions for half-pixel precision (0,1/2), (1/2,0) and (1/2,1/2). Thus, an individual CNN is trained for each fractional-pixel positions. Fig.12 shows a training example for three half-pixel CNN models. In essence, the principle of FRCNN is the same with that of adaptive interpolation filters [85], the parameters of which are derived by minimizing the prediction errors at fractional-pixel positions and need to be transmitted to decoder side. The performance of FRCNN

Fig. 13. Illustration for the proposed DVRF mode in [86].

TABLE II

THE CODING PERFORMANCEOF CNN-BASED BI-PREDICTIONAND DVRF

mainly thanks to the high prediction efficiency of CNN, and it achieves on average 3.9%, 2.7% and 1.3% bitrate saving compared to HM-16.7, under Low-Delay P (LDP), LowDelay B (LDB) and Random-Access (RA) configurations, respectively. However, the performance improvement comes from up to 120 FRCNN models for different slice types and 4 common QPs, which are trained from the specific videos compressed by HEVC under 4 common QPs and various coding configurations. Due to the poor generalization of the CNN models, the performance of FRCNN model may degenerate when applying it to the videos compressed by different configurations and QPs from training data, which is a potential problem to be solved in future.

Instead of improving the prediction performance for fractional-pixels, we directly explore the inter prediction block generation using CNN based frame rate up conversion (FRUC) techniques. A CNN based FRUC method in CTU level is proposed to generate a virtual reference frame FVirtual, which is utilized as a new reference frame and named as direct virtual reference frame (DVRF) [86], [87]. As shown in Fig. 13, current coding block can directly take the co-located block in FVirtual as inter prediction block without motion vectors. The state-of-the-art deep FRUC approach Adaptive Separable Convolution [88] is adopted and the two nearest bi-directional reference frames in the reference list are utilized as input for the network. This method achieves very promising compression performance, about 4.6% bitrate saving compared with HM-16.9 and 0.7% bitrate saving compared with JEM-7.1 [89] on average as shown in Table II. Herein, JEM (Joint Exploration Model) is the reference software based on HEVC reference model for the JVET group, which is an organization working on the exploration of next generation video coding standard established by ITU-T Video Coding Experts Group (VCEG) and the ISO/IEC Moving Picture Experts Group (MPEG) in Oct. 2015. In addition, considering the limitation of the traditional bidirectional prediction using simple average of two prediction hypothesises, we further improve its efficiency by utilizing a six-layer CNN with 13×13 receptive field size to infer the inter prediction block in a nonlinear fashion [90], [91], which achieves 2.7% bitrate saving compared with HM-16.19 on average under the RA configuration as shown in Table II. Although these methods obtained significant compression performance improvement, they also dramatically increase the run time for both encoding and decoding. The encoding and decoding time in Table II are all tested with GPU for the convolution calculations in the proposed methods. The computation efficiency is still a severe problem for CNN based video compression techniques in practical applications. The CNN based fractional interpolation methods can be referred to [92], [93].

C. Neural Network Based Quantization and Entropy Coding for Video Coding

In video coding, quantization and entropy coding are the lossy and lossless compression procedures respectively. For quantization part, the scalar quantization strategy has dominated hybrid video coding framework due to its low cost in computation and memory. However, this uniform scalar quantization does not conform to the characteristics of human visual system, and is not friendly to perceptual quality improvement. In [94], Alam et al. proposed a twostep quantization strategy using neural networks. In the first step, a CNN named VNet-2 is utilized to predict local visibility threshold CT for HEVC distortions of individual video frames, and the VNet-2 consists of 894 trainable parameters in three layers, i.e., convolution, subsampling and full connection, each of which contains one feature map. In the second stage, the quantization steps for 64 × 64 CTU are derived by regression as,

log= α+ β+ γ, (9)(Qstep) CT2 CT

where CT is predicted visibility threshold in the first stage, and {α,β,γ} are the model parameters related with patch features. The model parameters are predicted from three separate committees of neural networks respectively, and each committee had a total of five two-layered feed-forward networks with 10 neurons. Based on the proposed adaptive quantization strategy, on average 11% bitrate saving can be obtained for luma channel against HEVC at the same perceptual quality measured by structural similarity (SSIM) [95].

After quantization, the syntax elements including coding modes and transform coefficients will be fed into entropy coding engine to further remove their statistical redundancy. HEVC adopts the CABAC as its entropy coding, which achieves very high coding efficiency mainly because there are many contexts designed to predict conditional probabilities accurately. Inspired by the prediction efficiency of CNN, Song et al. improved the CABAC performance on compressing the syntax elements of 35 intra prediction modes by leveraging CNN to directly predict the probability distribution of intra modes instead of the handcrafted context models [96]. The network architecture is based on LeNet-5 proposed by LeCun et al. [97], and the above-left, the above and the left reconstructed blocks with the same size of current coding block are utilized as one category of inputs. The other category inputs are the most probable modes (MPMs), each of which are transformed into a 35-dim one-hot binary vector. The output is a 35-dim vector recording the predicted probability values of the 35 modes. Due to the high prediction accuracy, the CNN based method can improve the CABAC performance achieving about 9.0% bitrate saving for intra prediction mode coding when CU size is 8 × 8. The similar principle can be applied to other syntax elements, e.g., motion vector, coefficients and transform indices. Puri et al. applied the CNN to predict the optimal transform index probability distribution from the quantized coefficient blocks, and then utilized the probability to binarize the transform index using a variable length instead of a fixed length coding to improve the entropy coding performance [98]. At present, the works on entropy coding are still limited and remain to be investigated, especially there are few CNN based work on the dominant syntax, quantized transform coefficients in video coding, which may bring more coding gains.

D. Neural Network Based Loop Filtering

Loop filtering module is first introduced into video coding standard since H.263+ [99], and many different kinds of loop filters [21]–[23], [27], [28] are proposed after that. Especially, inspired by the success of CNN on image/video restoration filed, many of CNN based loop filters are designed to remove compression artifacts recently, which are much easier to implement the end-to-end training compared with other video coding modules. Zhang et al. [100] proposed a residual highway convolutional neural network (RHCNN) for loop filtering in HEVC. It is a deep network with 13 layers and the basic high way unit in each layer consists of two convolutional layers followed by the correponding activation function ReLUs and an identity skip connection. Since the compression noise levels are distinct for videos compressed with different QPs and frame types including I/B/P frames, the CNN models should be trained for different QP and frame type combinations, which lead to 156 CNN models for video coding application. To reduce memory cost for CNN based loop filters, Zhang et al. merged the QPs into several bands, and trained the optimal RHCNNs for each band. Compared with HM-12.0, the RHCNN achieves about 5.7%, 5.68% and 4.35% bitrate saving for I/P/B frames in low bit-rate circumstances respectively with 2∼3 times of encoding time increase even using GPU and 20∼30 times of encoding time increase using CPU.

By leveraging the coherence of the spatial and temporal adaptations, we improved the performance of CNN based loop filter, and designed the spatial-temporal residue network (STResNet) based loop filter [101]. The loss function of STResNet is formulated as,

N

L() = 1 F(x , x |) − y 2,                   (10)

N  i−1 i i i=1

represent thewhere −1−1are the training samples.and ireconstructed frames andx−1 and xy(xi, x(ii,yi))th th iii

TABLE III

THE CODING PERFORMANCE OF MUTI-MODEL CNN LOOP FILTER [102]

corresponds to the uncompressed ith frame. F−1|) represents the STResNet model, where is the set of network parameters. Moreover, we further improved the filtering performance by introducing content-aware CNN based loop filter in [102]. For a reconstructed frame, multiple CNN models are trained according to their filtering performance iteratively as that in [103], and a corresponding discriminative network is also trained which is utilized to help select optimal filter in test stage to remove coding overheads. Compared with HEVC with/whitout ALF under HEVC common test condition (CTC), the proposed multi-model CNN filters achieve significant performance improvement as illustrated in Table III at the cost of explosive encoding and decoding run time increase even using GeForce GTX TITAN X GPU.(xi, xi

Although the CNN based loop filters have achieved substantial coding gains on the top of HEVC, these methods need to store multiple CNN models for different QPs, which increase the memory burdens for video codec. In [104], they provided an efficient solution for CNN based loop filters with memory efficiency. They combined QPs as an input fed into the CNN training stage by simply padding the scalar QPs into a matrix with the same size of input frames or patches. To some extent, this method alleviates the performance fluctuates of CNN based loop filters due to QP missing in training stage. Based on our experience, although the CNN based loop filters learned from combined QPs is a little inferior to QP-dependent CNN models, the performance loss is usually marginal. The residual prediction based CNN model of in-loop filter is proposed in [105] and the multi-scale CNN model for in-loop filter is designed in [106].

Regarding the complexity of DL and none-DL based loop filtering methods under HEVC framework, the encoding time of [102] is 114% and 108% when the ALF is turned off/on respectively. However, the deocding time is drastically increased into 15010% and 12800% respectively. While the corresponding encoding and decoding complexity for ALF itself is 104% and 123%. Hence, there still exists large quantity of space and potential in optimization for DL based loop filtering algorithms in future studies, such as pruning and quantization for the float-point weights and biases in neural networks.

Besides the in-loop filters, there are also some postfiltering algorithms proposed to improve the quality of decoded video and images by reducing the compression artifacts. Dong et al. proposed an end-to-end CNN [107] to remove the compression artifacts, which is learned in the supervised manner. The CNN architecture is work is derived from superresolution network SRCNN [108] by embedding one or more “feature enhancement” layers after the first layer of SRCNN to clean the noisy features. Li et al. proposed a universal model to deal with compressed image at different compression ratios [109] by utilizing a very deep CNN model. Yang et al. proposed a multi-frame quality enhancement neural network for compressed video by utilizing the neighboring high quality frames to enhance the low quality frames. Herein, a support vector machine based detector is utilized to locate peak quality frames in compressed video [110]. CNN based quality enhancement also achieves convincing performance in the field of multiview plus depth video coding. Zhu et al. designed CNN models for the post-processing of synthesized views to promote the 3D video coding performances [111]. More works utilized more complicated structure to improve the compressed images [112], [113].

E. New Video Coding Frameworks Based on Neural Network

Although the elaborately designed hybrid video coding framework has achieved significant success on predominant compression performance, it becomes more and more difficult to be further improved. Moreover, it also becomes computation intensive and inhospitality to parallel computation as well as hardware manufacturer. Similar with neural network based image coding frameworks, some novel video coding frameworks are also investigated by assembling different neural network models. Chen et al. proposed a combination of several CNN networks called DeepCoder which achieved similar perceptual quality with low-profiled x264 encoder [114]. In DeepCoder, the intra prediction is implemented via a neural network to generate a feature map, denoted as fMap, and the inter prediction is obtained from motion estimation on previous frames. The fMap is further quantized and encoded into stream. The intra- and inter-prediction residuals are transformed into a more compact domain using neural networks, the process of which is similar with that of fMap generation in intra prediction but with different neural network parameters. Both the fMaps from intra prediction and residuals are quantized and coded using Huffman entropy coding. Although there are not as many coding tools as H.264/AVC, the DeepCoder shows comparable compression performance compared with H.264/AVC, which shows a new solution for video coding.

Chen et al. proposed a fully learning-based video coding framework by introducing the concept of VoxelCNN via exploring spatial-temporal coherence to effectively perform predictive coding inside learning network [115]. Specifically, the proposed video coding framework can be divided into three modules, i.e., predictive coding, iterative analysis/ synthesis and binarization. The VoxelCNN is designed to predict blocks in the video sequences conditioned on previously coded frames as well as the neighboring reconstructed blocks of current block. Then the compact discrete representation of the difference between predicted and original signals can be analyzed and synthesized in iterative manner using RNN model of Toderici et al. [63], which is composed of several LSTM-based auto-encoders with connections between adjacent stages. Finally, the bitstream is subsequently obtained after binarization and entropy coding. Although lack of entropy coding in their present work, the scheme still shows comparable performance with H.264/AVC, showing its potential in future video coding.

Inspired by the prediction for future frames of generative models [116], Srivastava et al. proposed to utilize the Long Short Term Memory (LSTM) Encoder-Decoder framework to learn video representations in [117], which can be utilized to predict future video frames. There are mainly two models, LSTM Autoencoder Model and LSTM Future Predictor Model, which consist of two recurrent neural networks. Different from Ranzato's work [116] predicting one future frame, this model can predict a long future sequence into the future. Based on the experiments, with 16 input natural video frames, the model can reconstruct these 16 frames and predict the future 13 frames.

V. OPTIMIZATION TECHNIQUES FOR IMAGE AND VIDEO COMPRESSION

The state-of-the-art video coding standard, HEVC, achieves the optimal compression performance by exhaustively traverse all the possible coding modes and partitions to determine the optimal coding parameters according to rate-distortion costs. The computational costs can be extremely reduced by predicting the optimal coding parameters to skip unnecessary RD calculations. The fast mode-decision algorithms are proposed for coding unit (CU) and prediction unit (PU) respectively on basis of neural networks, which are not only parallel-friendly but also easy for VLSI design [118], [119]. More specifically, the fast algorithm first carries out a coarse analysis based on the local gradients to classify the blocks into homogeneous and edge categories. This strategy not only can reduce the burden of CNN but also can make CNN avoid ill-conditions due to homogeneous blocks. Then, the CNN is designed for edge blocks to decrease no less than two CU partition modes in each CTU for full rate-distortion optimization process. The designed network contains one convolution layer with one max pooling layer followed by three full connected layers and takes the QP values into network at the last fully connected layer. Each square CU is used as network input while the output is the binary decision of quad-split or no-split for current CU. As such, the recursive mode traverse and selection process is eliminated. On average, their method achieves 61.1% intra coding time saving, whereas the BD rate loss is only 2.67% compared with HM-12.0. Xu et al. predicted the entire CTU partition structure by using both CNN and LSTM to determine whether the mode decision should be early terminated [120].

VI. CONCLUSIONS AND OUTLOOK

Image and video compression aims to seek more compact representation for visual signals while keeping high quality, and become more and more important in big visual data era. In this paper, the neural network based image and video compression techniques have been reviewed, especially for the recent deep learning based image and video compression techniques. With the survey presented earlier in this paper, it is apparent that the state-of-the-art neural network based end-to-end image compression is still in its infancy which only outperforms the JPEG2000 and struggles against HEVC. The marriage of neural network and traditional hybrid video coding framework obtained significant performance improvement compared with the latest video coding standard, HEVC. This demonstrates the advantages of both neural networks and hybrid video coding framework.

Based on the review, we think that the advantages of neural network in image and video compression are three folds. First, the excellent content adaptivity of neural network is superior to signal processing based model because the network parameters are derived based on lots of practical data while the models in the state-of-the-art coding standards are handcrafted based on image and video prior knowledge. Second, the larger receptive field is widely utilized in neural network models which not only utilizes the neighboring information but also can improve coding efficiency by leveraging samples from far distance, but the traditional coding tools only utilized the neighboring samples and are difficult to utilize far distant samples. Third, the neural network can well represent both texture and feature, which makes the joint compression optimization for both human view and machine vision analysis. However, the existing coding standards only pursue high compression performance toward human view task.

We envision that deep learning based image/video compression will play more important roles in representing and delivering images and videos with better quality and fewer bitrates, and the following confronted issues are required to be further investigated:

• Along with the fast development of computer vision techniques and explosively increasing of images and videos, the visual signal receivers are not only human visual system, but also the computer vision algorithms. Meanwhile, the neural network especially deep learning techniques are more appropriate for sematic information representation based on its great success in image and video understanding tasks. Therefore, the sematic-fidelity will become critical for further applications as well as traditional visual-fidelity requirement.Semantic-fidelity oriented image and video compression.

• The rate-distortion theory is the key of the success for traditional image and video compression, but it has not been well explored in current neural network based compression tasks. A single network to deal with all the images and videos with diverse structures is inefficient obviously. Therefore, the multi-network adaptively training and switching according to RD is a possible solution. • The biggest obstacle in hindering the deployment of deep learning based image and video compression is the burdens in computation and memory. To achieve high performance, larger neural networks with more layers and nodes are usually considered, but the various efficiency of network parameters are not well explored. For image and video compression problem, at present, there is no related research work by jointly considering both the compression performance and the efficiency in computation and memory for neural networks, which is important for practical applications.Rate-distortion (RD) optimization guided neural network training and adaptive switching for compression task. Memory and computation efficient design for practical image and video codec.

For the semantic-friendlily oriented image and video compression, we have attempted to design innovative visual signal representation framework to elegantly support both human vision viewing and machine vision analysis. In view of the lightweight and the importance of features for visual semantic descriptors, e.g., CNN features, we proposed the hierarchical visual signal representation in [121] by jointly compressing the feature descriptors and visual content. More specially, for each video frame, feature descriptors are first extracted and compressed, and then the decoded features are utilized to assist visual content compression by handling large-scale global motion. This strategy not only improves the visual content compression efficiency but also ensures the visual analysis performance due to the feature extraction from original video without the influence of compression artifacts. In [122], we further investigated the novel visual signal representation structure with deep learning based endto-end image compression framework, which can directly conduct more image understanding tasks from the compression domain. The rationale behind this approach lies in that the neural network architectures commonly used for learned compression (in particular the encoders) are similar to the ones commonly used for inference, and learned image encoders are hence, in principle, capable of extracting features relevant for inference tasks. As such, this approach could be extended in the future to simultaneously train and learn for the end-to-end image compression and understanding.

In CNN based image and video compression, the CNN model compression is also a multi-variable optimization problem, which should be optimized jointly considering computational cost, CNN performance and rates utilized for CNN transmission (if needed). The previous work [123] proposed a complexity-distortion optimization formulation under power constraints for video coding problem, which can be further extended to CNN model compression optimization jointly with computational costs and video compression performance.

Based on the discussion of this paper, neural network has also shown promising results on future image and video compression tasks. Although there are still many problems in computational complexity and memory consumption, their high efficiency in prediction and compact representation for image and video signals has made neural network obtain substantial coding gain on top of the state-of-the-art video coding frameworks. Their intrinsic parallel-friendly attribute also makes them suitable for the largely deployed parallel computation architectures, e.g., GPU and TPU. Moreover, the network based end-to-end optimization approaches are more flexible than hand-crafted methods, and they can be rapidly optimized or tuned, which also makes the network with enormous potential in further image and video compression problem as well as other artificial intelligence problems.

REFERENCES

[1] D. A. Huffman, “A method for the construction of minimum-redundancy codes,” Proc. Inst. Radio Eng., vol. 40, no. 9, pp. 1098–1101, Sep. 1952.

[2] S. Golomb, “Run-length encodings (corresp.),” IEEE Trans. Inf. Theory, vol. 12, no. 3, pp. 399–401, Jul. 1966.

[3] I. H. Witten, R. M. Neal, and J. G. Cleary, “Arithmetic coding for data compression,” Commun. ACM, vol. 30, no. 6, pp. 520–540, Jun. 1987.

[4] H. Andrews and W. Pratt, “Fourier transform coding of images,” in Proc. Hawaii Int. Conf. Syst. Sci., 1968, pp. 677–679.

[5] W. K. Pratt, J. Kane, and H. C. Andrews, “Hadamard transform image coding,” Proc. IEEE, vol. 57, no. 1, pp. 58–68, Jan. 1969.

[6] N. Ahmed, T. Natarajan, and K. R. Rao, “Discrete cosine transform,” IEEE Trans. Comput., vol. 100, no. 1, pp. 90–93, Jan. 1974.

[7] C. W. Harrison, “Experiments with linear prediction in television,” Bell System Tech. J., vol. 31, no. 4, pp. 764–783, Jul. 1952.

[8] G. K. Wallace, “Overview of the JPEG (ISO/CCITT) still image compression standard,” Image Process. Algorithms Techn., vol. 1244, pp. 220–234, Jun. 1990.

[9] C. Christopoulos, A. Skodras, and T. Ebrahimi, “The JPEG2000 still image coding system: An overview,” IEEE Trans. Consum. Electron., vol. 46, no. 4, pp. 1103–1127, Nov. 2000.

[10] D. Taubman, “High performance scalable image compression with EBCOT,” IEEE Trans. Image Process., vol. 9, no. 7, pp. 1158–1170, Jul. 2000.

[11] Y. Taki, M. Hatori, and S. Tanaka, “Interframe coding that follows the motion,” in Proc. Inst. Electron. Commun. Eng. Jpn. Annu. Conv.(IECEJ), Aug. 1974, p. 1263.

[12] A. N. Netravali and J. A. Stuller, “Motion-compensated transform coding,” Bell Syst. Tech. J., vol. 58, no. 7, pp. 1703–1718, Sep. 1979.

[13] C. Reader, History of Video Compression (Draft), document JVT-D068, 2002.

[14] T. Wiegand, G. J. Sullivan, G. Bjøntegaard, and A. Luthra, “Overview of the H.264/AVC video coding standard,” IEEE Trans. Circuits Syst. Video Technol., vol. 13, no. 7, pp. 560–576, Jul. 2003.

[15] (2018). AVS Working Group Website. [Online]. Available: http://www.avs.org.cn

[16] G. J. Sullivan, J.-R. Ohm, W.-J. Han, and T. Wiegand, “Overview of the high efficiency video coding (HEVC) standard,” IEEE Trans. Circuits Syst. Video Technol., vol. 22, no. 12, pp. 1649–1668, Dec. 2012.

[17] H. Lv, R. Wang, X. Xie, H. Jia, and W. Gao, “A comparison of fractional-pel interpolation filters in HEVC and H. 264/AVC,” in Proc. Vis. Commun. Image Process. (VCIP), Nov. 2012, pp. 1–6.

[18] J. Lainema, F. Bossen, W.-J. Han, J. Min, and K. Ugur, “Intra coding of the HEVC standard,” IEEE Trans. Circuits Syst. Video Technol., vol. 22, no. 12, pp. 1792–1801, Dec. 2012.

[19] J.-L. Lin, Y.-W. Chen, Y.-W. Huang, and S.-M. Lei, “Motion vector coding in the HEVC standard,” IEEE J. Sel. Topics Signal Process., vol. 7, no. 6, pp. 957–968, Dec. 2013.

[20] M. Naccari and F. Pereira, “Adaptive bilateral filter for improved in-loop filtering in the emerging high efficiency video coding standard,” in Proc. IEEE Picture Coding Symp. (PCS), May 2012, pp. 397–400.

[21] X. Zhang et al., “Low-rank-based nonlocal adaptive loop filter for high-efficiency video compression,” IEEE Trans. Circuits Syst. Video Technol., vol. 27, no. 10, pp. 2177–2188, Oct. 2017.

[22] S. Ma, X. Zhang, J. Zhang, C. Jia, S. Wang, and W. Gao, “Nonlocal in-loop filter: The way toward next-generation video coding?” IEEE MultiMedia, vol. 23, no. 2, pp. 16–26, Apr./Jun. 2016.

[23] C.-Y. Tsai et al., “Adaptive loop filtering for video coding,” IEEE J. Sel. Topics Signal Process., vol. 7, no. 6, pp. 934–945, Dec. 2013.

[24] X. Zhang, R. Xiong, S. Ma, and W. Gao, “Adaptive loop filter with temporal prediction,” in Proc. IEEE Picture Coding Symp. (PCS), May 2012, pp. 437–440.

[25] X. Zhang, S. Wang, Y. Zhang, W. Lin, S. Ma, and W. Gao, “Highefficiency image coding via near-optimal filtering,” IEEE Signal Process. Lett., vol. 24, no. 9, pp. 1403–1407, Sep. 2017.

[26] P. List, A. Joch, J. Lainema, G. Bjontegaard, and M. Karczewicz, “Adaptive deblocking filter,” IEEE Trans. Circuits Syst. Video Technol., vol. 13, no. 7, pp. 614–619, Jul. 2003.

[27] A. Norkin et al., “HEVC deblocking filter,” IEEE Trans. Circuits Syst. Video Technol., vol. 22, no. 12, pp. 1746–1754, Dec. 2012.

[28] C.-M. Fu et al., “Sample adaptive offset in the HEVC standard,” IEEE Trans. Circuits Syst. Video Technol., vol. 22, no. 12, pp. 1755–1764, Dec. 2012.

[29] G. E. Hinton, “Learning translation invariant recognition in a massively parallel networks,” in Proc. Int. Conf. Parallel Archit. Lang. Eur., 1987, pp. 1–13.

[30] F. Rosenblatt, Principles of Neurodynamics: Perceptrons and the Theory of Brain Mechanisms. Washington, DC, USA: Spartan Books, 1962.

[31] P. Werbos, “New tools for prediction and analysis in the behavioral sciences,” Ph. D. dissertation, Dept. Statist., Harvard Univ., Cambridge, MA, USA, 1974.

[32] D. E. Rumelhart, G. E. Hinton, and R. J. Williams, “Learning representations by back-propagating errors,” Nature, vol. 323, no. 6088, p. 533, 1986.

[33] Y. Le Cun et al., “Handwritten zip code recognition with multilayer networks,” in Proc. 10th Int. Conf. Pattern Recognit., Jun. 1990, pp. 35–40.

[34] G. K. Wallace, “The JPEG still picture compression standard,” Commun. ACM, vol. 34, no. 4, pp. 30–44, 1991.

[35] L. O. Chua and T. Lin, “A neural network approach to transform image coding,” Int. J. Circuit Theory Appl., vol. 16, no. 3, pp. 317–324, 1988. [36] M. W. Gardner and S. R. Dorling, “Artificial neural networks (the multilayer perceptron)-a review of applications in the atmospheric sciences,” Atmos. Environ., vol. 32, nos. 14–15, pp. 2627–2636, Aug. 1998.

[37] R. J. Schalkoff, Artificial Neural Networks, vol. 1. New York, NY, USA: McGraw-Hill, 1997.

[38] N. Sonehara, M. Kawato, S. Miyake, and K. Nakane, “Image data compression using a neural network model,” in Proc. IJCNN, vol. 2, 1989, pp. 35–41.

[39] P. Munro and D. Zipser, “Image compression by back propagation: an example of extensional programming,” Models cognition: rev. Cognit. Sci., vol. 1, no. 208, p. 1, 1989.

[40] G. L. Sicuranza, G. Romponi, and S. Marsi, “Artificial neural network for image compression,” Electron. Lett., vol. 26, no. 7, pp. 477–479, Mar. 1990.

[41] R. D. Dony and S. Haykin, “Neural network approaches to image compression,” Proc. IEEE, vol. 83, no. 2, pp. 288–303, Feb. 1995.

[42] S. Dianat, N. Nasrabadi, and S. Venkataraman, “A non-linear predictor for differential pulse-code encoder (DPCM) using artificial neural networks,” in Proc. Int. Conf. Acoust., Speech, Signal Process., Apr. 1991, pp. 2793–2796.

[43] C. N. Manikopoulos, “Neural network approach to DPCM system design for image coding,” IEE Proc. I Commun., Speech Vis., vol. 139, no. 5, pp. 501–507, Oct. 1992.

[44] A. Namphol, S. H. Chin, and M. Arozullah, “Image compression with a hierarchical neural network,” IEEE Trans. Aerosp. Electron. Syst., vol. 32, no. 1, pp. 326–338, Jan. 1996.

[45] J. G. Daugman, “Complete discrete 2-D Gabor transforms by neural networks for image analysis and compression,” IEEE Trans. Acoust., Speech, Signal Process., vol. 36, no. 7, pp. 1169–1179, Jun. 1988.

[46] H. M. Abbas and M. M. Fahmy, “Neural model for Karhunen-Loeve transform with application to adaptive image compression,” IEE Proc. I, Commun., Speech Vis., vol. 140, no. 2, pp. 135–143, Apr. 1993.

[47] E. Gelenbe, “Random neural networks with negative and positive signals and product form solution,” Neural Comput., vol. 1, no. 4, pp. 502–510, Dec. 1989.

[48] E. Gelenbe and M. Sungur, “Random network learning and image compression,” in Proc. IEEE Int. Conf. Neural Netw. (ICNN), vol. 6, Jun. 1994, pp. 3996–3999.

[49] C. Cramer, E. Gelenbe, and I. Bakircioglu, “Video compression with random neural networks,” in Proc. Int. Workshop Neural Netw. Identificat., Control, Robot. Signal/Image Process., Aug. 1996, pp. 476–484.

[50] F. Hai, K. F. Hussain, E. Gelenbe, and R. K. Guha, “Video compression with wavelets and random neural network approximations,” Proc. SPIE, vol. 4305, pp. 57–65, Aug. 2001.

[51] Y. LeCun, Y. Bengio, and G. Hinton, “Deep learning,” Nature, vol. 521, no. 7553, p. 436, 2015.

[52] J. Ballé, V. Laparra, and E. P. Simoncelli. (2016). “End-to-end optimized image compression.” [Online]. Available: https://arxiv. org/abs/1611.01704

[53] J. Ballé, V. Laparra, and E. P. Simoncelli, “End-to-end optimization of nonlinear transform codes for perceptual quality,” in Proc. Picture Coding Symp. (PCS), Dec. 2016, pp. 1–5.

[54] J. Ballé, D. Minnen, S. Singh, S. J. Hwang, and N. Johnston, “Variational image compression with a scale hyperprior,” in Proc. Int. Conf. Learn. Represent., May 2018, pp. 1–10.

[55] D. Minnen, J. Ballé, and G. D. Toderici, “Joint autoregressive and hierarchical priors for learned image compression,” in Proc. Adv. Neural Inf. Process. Syst., 2018, pp. 10771–10780.

[56] L. Zhou, C. Cai, Y. Gao, S. Su, and J. Wu, “Variational autoencoder for low bit-rate image compression,” in Proc. IEEE Conf. Comput. Vis.

Pattern Recognit. Workshops, Aug. 2018, pp. 2617–2620.

[57] E. Agustsson et al., “Soft-to-hard vector quantization for end-toend learning compressible representations,” in Proc. Adv. Neural Inf. Process. Syst., 2017, pp. 1141–1151.

[58] L. Theis, W. Shi, A. Cunningham, and F. Huszär. (2017). “Lossy image compression with compressive autoencoders.” [Online]. Available: https://arxiv.org/abs/1703.00395

[59] E. Ahanonu, M. Marcellin, and A. Bilgin, “Lossless Image Compression Using Reversible Integer Wavelet Transforms and Convolutional Neural Networks,” in Proc. IEEE Data Compress. Conf., Aug. 2018, pp. 1–10.

[60] S. Hochreiter and J. Schmidhuber, “Long short-term memory,” Neural Comput., vol. 9, no. 8, pp. 1735–1780, 1997.

[61] K. Cho, B. van Merriénboer, D. Bahdanau, and Y. Bengio. (2015). “On the properties of neural machine translation: Encoder-decoder approaches.” [Online]. Available: https://arxiv.org/abs/1409.1259

[62] J. Chung, C. Gulcehre, K. Cho, and Y. Bengio. (2014). “Empirical evaluation of gated recurrent neural networks on sequence modeling.” [Online]. Available: https://arxiv.org/abs/1412.3555

[63] G. Toderici et al., “Full resolution image compression with recurrent neural networks,” in Proc. CVPR, Apr. 2017, pp. 5435–5443.

[64] D. Minnen et al. (2018). “Spatially adaptive image compression using a tiled deep network.” [Online]. Available: https://arxiv. org/abs/1802.02629

[65] O. Rippel and L. Bourdev. (2017). “Real-time adaptive image compression.” [Online]. Available: https://arxiv.org/abs/1705.05823

[66] C. Jia, X. Zhang, S. Wang, S. Wang, S. Pu, and S. Ma, “Light field image compression using generative adversarial network based view synthesis,” IEEE J. Emerg. Sel. Topics Circuits Syst., to be published.

[67] K. Gregor, I. Danihelka, A. Graves, D. J. Rezende, and D. Wierstra. (2015). “Draw: A recurrent neural network for image generation.”

[Online]. Available: https://arxiv.org/abs/1502.04623

[68] K. Gregor, F. Besse, D. J. Rezende, I. Danihelka, and D. Wierstra, “Towards conceptual compression,” in Proc. Adv. Neural Inf. Process. Syst., 2016, pp. 3549–3557.

[69] E. Agustsson, M. Tschannen, F. Mentzer, R. Timofte, and L. Van Gool, “Extreme learned image compression with GANs,” in Proc. IEEE Conf. Comput. Vis. Pattern Recognit. Workshops, Sep. 2018, pp. 2587–2590.

[70] J. Li, B. Li, J. Xu, R. Xiong, and W. Gao, “Fully connected networkbased intra prediction for image coding,” IEEE Trans. Image Process., vol. 27, no. 7, pp. 3236–3246, Jul. 2018.

[71] Y. Li et al., “A hybrid neural network for chroma intra prediction,” in Proc. 25th IEEE Int. Conf. Image Process. (ICIP), Oct. 2018, pp. 1797–1801.

[72] J. Pfaff et al., Intra Prediction Modes Based on Neural Networks, document JVET-J0037, Apr. 2018, pp. 1–14.

[73] Y. Li et al., “Convolutional neural network-based block up-sampling for intra frame coding,” IEEE Trans. Circuits Syst. for Technol., vol. 28, no. 9, pp. 2316–2330, Sep. 2017.

[74] J. Lin, D. Liu, H. Yang, H. Li, and F. Wu, “Convolutional neural network-based block Up-sampling for HEVC,” IEEE Trans. Circuits Syst. Video Technol., to be published.

[75] R. Molina, A. K. Katsaggelos, L. D. Alvarez, and J. Mateos, “Toward a new video compression scheme using super-resolution,” Proc. SPIE, vol. 6077, p. 607706, Jan. 2006.

[76] M. Shen, P. Xue, and C. Wang, “Down-sampling based video coding using super-resolution technique,” IEEE Trans. Circuits Syst. Video Technol., vol. 21, no. 6, pp. 755–765, Jun. 2011.

[77] J. Pfaff et al., “Neural network based intra prediction for video coding,” Proc. SPIE, vol. 10752, Aug. 2018, Art. no. 1075213.

[78] L. Feng, X. Zhang, X. Zhang, S. Wang, R. Wang, and S. Ma, “A dualnetwork based super-resolution for compressed high definition video,” in Pacific-Rim Conf. Multimedia. New York, NY, USA: Springer, 2018, pp. 600–610.

[79] Y. Li, D. Liu, H. Li, L. Li, Z. Li, and F. Wu, “Learning a convolutional neural network for image compact-resolution,” IEEE Trans. Image Process., vol. 28, no. 3, pp. 1092–1107, 2019.

[80] Z.-T. Zhang, C.-H. Yeh, L.-W. Kang, and M.-H. Lin, “Efficient CTU-based intra frame coding for HEVC based on deep learning,” in Proc. Asia-Pacific Signal Inf. Process. Assoc. Annu. Summit Conf., Dec. 2017, pp. 661–664.

[81] Y. Hu, W. Yang, S. Xia, W.-H. Cheng, and J. Liu, “Enhanced intra prediction with recurrent neural network in video coding,” in IEEE Data Compress. Conf. (DCC), 2018, p. 413.

[82] S. Huo, D. Liu, F. Wu, and H. Li, “Convolutional neural network-based motion compensation refinement for video coding,” in Proc. Int. Symp.

Circuits Syst. (ISCAS), May 2018, pp. 1–4.

[83] Y. Dai, D. Liu, and F. Wu, “A convolutional neural network approach for post-processing in HEVC intra coding,” in Proc. Int. Conf. Multimedia Model., 2017, pp. 28–39.

[84] N. Yan, D. Liu, H. Li, B. Li, L. Li, and F. Wu, “Convolutional neural network-based fractional-pixel motion compensation,” IEEE Trans. Circuits Syst. Video Technol., vol. 29, no. 3, pp. 840–853, Mar. 2018.

[85] Y. Vatis and J. Ostermann, “Adaptive interpolation filter for H.264/AVC,” IEEE Trans. Circuits Syst. Video Technol., vol. 19, no. 2, pp. 179–192, Feb. 2009.

[86] L. Zhao, S. Wang, X. Zhang, S. Wang, S. Ma, and W. Gao, “Enhanced CTU-level inter prediction with deep frame rate up-conversion For high efficiency video coding,” in Proc. 25th IEEE Int. Conf. Image Process. (ICIP), Mar. 2018, pp. 206–210.

[87] L. Zhao, “Enhanced motion-compensated video coding with deep virtual reference frame generation,” IEEE Trans. Image Process., to be published.

[88] S. Niklaus, L. Mai, and F. Liu. (2017). “Video frame interpolation via adaptive separable convolution.” [Online]. Available: https://arxiv.org/abs/1708.01692

[89] J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, and J. Boyce, Algorithm Description of Joint Exploration Test Model 1, document JVET-A1001, Oct. 2015, pp. 1–48.

[90] Z. Zhao, S. Wang, S. Wang, X. Zhang, S. Ma, and J. Yang, “CNN-based bi-directional motion compensation for high efficiency video coding,” in Proc. Int. Symp. Circuits Syst. (ISCAS), Aug. 2018, pp. 1–4.

[91] Z. Zhao, S. Wang, S. Wang, and J. Yang, “Enhanced Bi-prediction with convolutional neural network for high efficiency video coding,” IEEE Trans. Circuits Syst. Video Technol., to be published.

[92] H. Zhang, L. Song, Z. Luo, and X. Yang, “Learning a convolutional neural network for fractional interpolation in HEVC inter coding,” in Proc. Vis. Commun. Image Process. (VCIP), Dec. 2017, pp. 1–4.

[93] N. Yan, D. Liu, H. Li, T. Xu, F. Wu, and B. Li, “Convolutional neural network-based invertible half-pixel interpolation filter for video coding,” in Proc. 25th IEEE Int. Conf. Image Process. (ICIP), Aug. 2018, pp. 201–205.

[94] M. M. Alam, T. D. Nguyen, M. T. Hagan, and D. M. Chandler,

“A perceptual quantization strategy for HEVC based on a convolutional neural network trained on natural images,” Proc. SPIE, vol. 9599, Aug. 2015, Art. no. 959918.

[95] Z. Wang, A. C. Bovik, H. R. Sheikh, and E. P. Simoncelli, “Image quality assessment: From error visibility to structural similarity,” IEEE Trans. Image Process., vol. 13, no. 4, pp. 600–612, Apr. 2004.

[96] R. Song, D. Liu, H. Li, and F. Wu, “Neural network-based arithmetic coding of intra prediction modes in HEVC,” in Proc. Vis. Commun. Image Process. (VCIP), Feb. 2017, pp. 1–4.

[97] Y. LeCun, L. Bottou, Y. Bengio, and P. Haffner, “Gradient-based learning applied to document recognition,” Proc. IEEE, vol. 86, no. 11, pp. 2278–2324, Nov. 1998.

[98] S. Puri, S. Lasserre, and P. L. Callet, “CNN-based transform index prediction in multiple transforms framework to assist entropy coding,” in Proc. Eur. Signal Process. Conf. (EUSIPCO), Feb. 2017, pp. 798–802.

[99] G. Cote, B. Erol, M. Gallant, and F. Kossentini, “H.263+: Video coding at low bit rates,” IEEE Trans. Circuits Syst. for Video Technol., vol. 8, no. 7, pp. 849–866, Nov. 1998.

[100] Y. Zhang, T. Shen, X. Ji, Y. Zhang, R. Xiong, and Q. Dai, “Residual highway convolutional neural networks for in-loop filtering in HEVC,” IEEE Trans. Image Process., vol. 27, no. 8, pp. 3827–3241, Aug. 2018.

[101] C. Jia, S. Wang, X. Zhang, S. Wang, and S. Ma, “Spatial-temporal residue network based in-loop filter for video coding,” in Proc. Vis. Commun. Image Process. (VCIP), Mar. 2017, pp. 1–4.

[102] C. Jia et al., “Content-aware convolutional neural network for in-loop filtering in high efficiency video coding,” IEEE Trans. Image Process., to be published.

[103] X. F. Zhang, S. Wang, K. Gu, W. Lin, S. Ma, and W. Gao, “Justnoticeable difference-based perceptual optimization for JPEG compression,” IEEE Signal Process. Lett., vol. 24, no. 1, pp. 96–100, Jan. 2017.

[104] X. Song et al. (2018). “A practical convolutional neural network as loop filter for intra frame.” [Online]. Available: https://arxiv.org/ abs/1805.06121

[105] W.-S. Park and M. Kim, “CNN-based in-loop filtering for coding efficiency improvement,” in Proc. IEEE 12th Image, Video, Multidimensional Signal Process. Workshop, Jun. 2016, pp. 1–5.

[106] J. Kang, S. Kim, and K. M. Lee, “Multi-modal/multi-scale convolutional neural network based in-loop filter design for next generation video codec,” in Proc. Int. Conf. Image Process. (ICIP), Sep. 2017, pp. 26–30.

[107] C. Dong, Y. Deng, C. Change Loy, and X. Tang, “Compression artifacts reduction by a deep convolutional network,” in Proc. IEEE Int. Conf. Comput. Vis., May 2015, pp. 576–584.

[108] C. Dong, C. C. Loy, K. He, and X. Tang, “Learning a deep convolutional network for image super-resolution,” in Proc. Eur. Conf. Comput. Vis., 2014, pp. 184–199.

[109] K. Li, B. Bare, and B. Yan, “An efficient deep convolutional neural networks model for compressed image deblocking,” in Proc. Int. Conf. Multimedia Expo (ICME), Jul. 2017, pp. 1320–1325.

[110] R. Yang, M. Xu, Z. Wang, and T. Li, “Multi-frame quality enhancement for compressed video,” in Proc. IEEE Conf. Comput. Vis. Pattern Recognit., Feb. 2018, pp. 6664–6673.

[111] L. Zhu, Y. Zhang, S. Wang, H. Yuan, S. Kwong, and H. H.-S. Ip,

“Convolutional neural network-based synthesized view quality enhancement for 3D video coding,” IEEE Trans. Image Process., vol. 27, no. 11, pp. 5365–5377, Nov. 2018.

[112] L. Cavigelli, P. Hager, and L. Benini, “CAS-CNN: A deep convolutional neural network for image compression artifact suppression,” in Proc. Int. Joint Conf. Neural Netw. (IJCNN), May 2017, pp. 752–759.

[113] B. Zheng, R. Sun, X. Tian, and Y. Chen, “S-Net: a scalable convolutional neural network for JPEG compression artifact reduction,” J. Electron. Imag., vol. 27, no. 4, 2018, Art. no. 043037.

[114] T. Chen, H. Liu, Q. Shen, T. Yue, X. Cao, and Z. Ma, “DeepCoder: A deep neural network based video compression,” in Proc. Vis. Commun. Image Process. (VCIP), Dec. 2017, pp. 1–4.

[115] Z. Chen, T. He, X. Jin, and F. Wu. (2018). “Learning for video compression.” [Online]. Available: https://arxiv.org/abs/1804.09869

[116] M. Ranzato, A. Szlam, J. Bruna, M. Mathieu, R. Collobert, and S. Chopra. (2014). “Video (language) modeling: a baseline for generative models of natural videos.” [Online]. Available: https://arxiv.org/ abs/1412.6604

[117] N. Srivastava, E. Mansimov, and R. Salakhudinov, “Unsupervised learning of video representations using LSTMS,” in Proc. Int. Conf. Mach. Learn., 2015, pp. 843–852.

[118] Z. Liu, X. Yu, Y. Gao, S. Chen, X. Ji, and D. Wang, “CU partition mode decision for HEVC hardwired intra encoder using convolution neural network,” IEEE Trans. Image Process., vol. 25, no. 11, pp. 5088–5103, Nov. 2016.

[119] N. Song, Z. Liu, X. Ji, and D. Wang, “CNN oriented fast PU mode decision for HEVC hardwired intra encoder,” in Proc. IEEE Global Conf. Signal Inf. Process. (GlobalSIP), Feb. 2017, pp. 239–243.

[120] M. Xu, T. Li, Z. Wang, X. Deng, R. Yang, and Z. Guan, “Reducing complexity of HEVC: A deep learning approach,” IEEE Trans. Image Process., to be published.

[121] X. Zhang, S. Ma, S. Wang, X. Zhang, H. Sun, and W. Gao, “A joint compression scheme of video feature descriptors and visual content,” IEEE Trans. Image Process., vol. 26, no. 2, pp. 633–647, Feb. 2017.

[122] Y. Li, C. Jia, X. Zhang, S. Wang, S. Ma, and W. Gao, “Joint rate-distortion optimization for simultaneous texture and deep feature compression of facial images,” in Proc. IEEE Int. Conf. Multimedia Big Data (BigMM), Aug. 2018, pp. 334–341.

[123] L. Su, Y. Lu, F. Wu, S. Li, and W. Gao, “Complexity-constrained H.264 video encoding,” IEEE Trans. Circuits Syst. Video Technol., vol. 19, no. 4, pp. 477–490, Apr. 2009.

Siwei Ma (M&apos;12) received the B.S. degree from Shandong Normal University, Jinan, China, in 1999, and the Ph.D. degree in computer science from the Institute of Computing Technology, Chinese Academy of Sciences, Beijing, China, in 2005. He held a post-doctoral position with the University of Southern California at Los Angeles, Los Angeles, CA, USA, from 2005 to 2007. He joined the School of Electronics Engineering and Computer Science, Institute of Digital Media, Peking University, Beijing, where he is currently a Professor. He has

authored more than 200 technical articles in refereed journals and proceedings in image and video coding, video processing, video streaming, and transmission. He is an Associate Editor of the IEEE TRANSACTIONS ON CIRCUITS AND SYSTEMS FOR VIDEO TECHNOLOGY and the Journal of

Visual Communication and Image Representation.

Xinfeng Zhang (M&apos;16) received the B.S. degree in computer science from the Hebei University of Technology, Tianjin, China, in 2007, and the Ph.D. degree in computer science from the Institute of Computing Technology, Chinese Academy of Sciences, Beijing,

China, in 2014. From 2014 to 2017, he was a Research Fellow with the Rapid-Rich Object Search Lab, Nanyang Technological University, Singapore. From 2017 to 2018, he was a Post-Doctoral Fellow with the School of Electrical Engineering System,

University of Southern California, Los Angeles, CA, USA. From 2018 to 2019, he was a Research Fellow with the Department of Computer Science, City University of Hong Kong. He is currently an Assistant Professor with the Department of Computer Science, University of Chinese Academy of Sciences. He has given more than 20 technical proposals to ISO/MPEG, ITU-T, and AVS standards, and has authored more than 100 refereed journals/conference papers. His research interests include video compression, image/video quality assessment, and image/video analysis. He received the Best Paper Award at the 2017 Pacific-Rim Conference on Multimedia, the Best Paper Award of IEEE Multimedia 2018, and is the coauthor of a paper that received the Best Student Paper Award in the IEEE International Conference on Image Processing 2018.

Chuanmin Jia (S&apos;18) received B.E. degree in computer science from the Beijing University of Posts and Telecommunications, Beijing, China, in 2015. He is currently pursuing the Ph.D. degree with the Department of Computer Science, Peking University, Beijing, China. He was a Visiting Student with the Video Lab, New York University, NY, USA, in 2018. His research interests include video compression, light field compression, and machine learning.

He received the Best Paper Award of the Pacific-Rim Conference on Multimedia in 2017 and the IEEE Multimedia in 2018, and the Best Student Paper Award in the IEEE International Conference on Multimedia Information Processing and Retrieval in 2019.

Zhenghui Zhao received the B.S. degree from the School of Mathematical Sciences, Dalian University of Technology, in 2015. He is currently pursuing the Ph.D. degree with Peking University. His research interests include image and video compression, and image and video processing.

Shiqi Wang received the B.S. degree in computer science from the Harbin Institute of Technology in 2008 and the Ph.D. degree in computer application technology from Peking University in 2014. From 2014 to 2016, he was a Post-Doctoral Fellow with the Department of Electrical and Computer Engineering, University of Waterloo, Waterloo,

Canada. From 2016 to 2017, he was with the Rapid-Rich Object Search Laboratory, Nanyang Technological University, Singapore, as a Research

Fellow. He is currently an Assistant Professor with

the Department of Computer Science, City University of Hong Kong. He has given more than 40 technical proposals to ISO/MPEG, ITU-T, and AVS standards, and has authored more than 150 refereed journals/conference papers. His research interests include video compression, image/video quality assessment, and image/video search and analysis. He received the Best Paper Award at the 2017 Pacific-Rim Conference on Multimedia, the Best Paper Award of IEEE Multimedia 2018, and is the coauthor of a paper that received the Best Student Paper Award in the IEEE International Conference on Image Processing 2018.

Shanshe Wang received the B.S. degree from the Department of Mathematics, Heilongjiang University, Harbin, China, in 2004, the M.S. degree in computer software and theory from Northeast Petroleum University, Daqing, China, in 2010, and the Ph.D. degree in computer science from the Harbin Institute of Technology. He held a post-doctoral position with Peking University from 2016 to 2018. He joined the School of Electronics Engineering and Computer Science, Institute of Digital Media, Peking

University, Beijing, where he is currently a Research Assistant Professor. His current research interests include video compression and image and video quality assessment.